Date: March 26, 2002

To: Cinema Technicians

From: QSC Audio Poducts Inc.

Subject: New Default Speaker Database

The DCM Commander software has a speaker database file with default speaker settings to get you started setting up your cinema sound system. The files accompanying this are the upgrade files with default settings for EAW, EV, JBL, KCS, Klipsch, Martin, SA and WSS. Every effort has been made to get adequate speaker data from the speaker manufactures. But from some companies the data was not accurate or non-existent. The crossover and sensitivity compensation (i.e. High, Mid Low gain) has been set. Driver delay (i.e. time alignment) has been set with manufactures data and should be checked onsite to verify a smooth response. The CD horn boost, Screen EQ and Parametric EQ’s have been left off. Use these to obtain the right amount of high frequency required in the auditorium and smooth frequency response.

Attached are two files Default.dbf and Default.mdx. You need to replace your current files with these ones. If you installed DCM Commander with its default settings the file path to the speaker database should be this.

C:\Program Files\QSC\DCM Commander v2.0

Just drag and drop or cut and paste the two files into this folder. You will get a message “This folder already contains a file named Default.dbf would you like to replace the existing file with this one?” Click YES. Or Yes to All if you are copying both files at once.

Please send your feedback to francois_godfrey@qscaudio.com

Thanks,

Francois Godfrey

QSC Audio Products Inc.

Cinema Product Specialist

1665 MacArthur Blvd.

Costa Mesa, CA 92626 USA

Phone:	(714) 754-6175

(800) 854-4079

Direct:	(714) 957-7133

Fax:		(714) 754-6174

francois_godfrey@qscaudio.com

http://www.qscaudio.com